# Project Plan: IoT Zero-Trust Hardware Vault

Objective: Proof of Concept for a multi-layered security system with MQTT logging.

Core Logic & Working Principle

The system operates on a State Machine to track what is happening in real-time.

- State 0 (IDLE): The outer door is closed. The LDR reads darkness. The system is quietly waiting.

- State 1 (TRIGGERED): The outer door is opened. Light hits the LDR.

  - The ESP32 starts a strict millis() timer (e.g., 15 seconds).

  - The LCD Display prompts: "ENTER CODE".

- State 2 (AUTHENTICATION): The user must turn the Potentiometer to a specific secret value and press the Switch to confirm.

- State 3A (SUCCESS): If the switch is pressed at the right value before 15 seconds, the LCD reads "ACCESS GRANTED", and an MQTT message logs a successful entry.

- State 3B (TIMEOUT / BREACH): If 15 seconds pass with no input, OR the wrong code is entered, the Buzzer sounds, a Red LED flashes, and an MQTT "ACCESS TIMEOUT / INTRUSION" alert is published to the cloud.

Implementation Guide: Scratch to End

Circuitry & Hardware Wiring

Keep the breadboard wiring clean to avoid short circuits.

- Input 1 (Outer Door): Connect the LDR to an analog pin using a voltage divider with a Resistor.

- Input 2 (Authentication Dial): Connect the Potentiometer's middle pin to another analog pin.

- Input 3 (Enter Button): Connect a Switch to a digital pin. Use the ESP32's internal pull-up resistor in your code (INPUT_PULLUP).

- Outputs: Connect the Buzzer and an LED to digital pins. Connect the LCD (using standard pins or I2C if available).

### Coding the Logic in IDE

Do not use delay() for your timeout feature, as it pauses the whole microcontroller and stops it from reading the potentiometer or switch. Use millis().

- Step 1: Sensor Test. Read the LDR and print it to the Serial Monitor to find the threshold value between "door closed" (dark) and "door open" (light).

- Commit 2: git commit -m "Added LDR threshold and basic inputs"

- Step 2: The Timer Logic. ```cpp if (LDR_Value > threshold && !timerStarted) { startTime = millis(); timerStarted = true; } if (timerStarted && (millis() - startTime >= 15000)) { triggerAlarm(); // 15 seconds passed! }

- Step 3: Add Wi-Fi & MQTT. Include the WiFi.h and PubSubClient.h libraries. Set up the connection to the public HiveMQ broker.

- Commit 3: git commit -m "Implemented millis timer and MQTT connection"

### Testing & Presentation Prep

- Testing: Open the HiveMQ public web socket client on your laptop. Trigger the LDR with your hand, wait 15 seconds, and watch the dashboard light up with your timeout alert.